U18CO018

Shubham Shekhaliya

Assignment-5 (MIT)

1-> A string of readings is stored in memory, locations starting at 2070H, and the end of the string is indicated by the byte 0DH.Write a program to check each byte in the string, and the save the bytes in the range of 30H to 39H (both inclusive) in memory locations starting from 2090H.

Code: -

LXI H, 2070H

LXI D, 2090H

MVI C, 0DH

Start: MOV A, M

CMP C

JZ End

MOV A, M

CPI 30H

JZ Done

JC Next

CPI 39H

JZ Done

JNC Next

Done: STAX D

INX D

Next: INX H

JMP Start

End: HLT

2-> A set of ten bytes is stored in memory starting with the address 2050H.Write a program to check each byte , and save the bytes that are higher than 6010 and lower than 10010 in memory locations starting from 2060H.

Code:-

LXI H, 2050H

LXI D, 2060H

MVI C, 10

Loop: MOV A, M

CPI 60

JZ Next

JC Next

CPI 100

JZ Next

JNC Next

STAX D

INX D

Next: INX H

DCR C

JNZ Loop

End: HLT

3-> Data bytes are stored in memory locations from 2050H to 205FH.To insert an additional five bytes of data, It is necessary to shift the data string by five memory locations. Write a program to store the data string from 2055H to 2064H.Use any sixteen bytes of data to verify your program.

MVI C, 10H

LXI D, 2064H

LXI H, 205FH

LOOP: MOV A, M

STAX D

DCX D

DCX H

DCR C

JNZ LOOP

; 0 at 2050H-2054H

MVI A, 0

MVI C, 5

Loop: STAX D

DCX D

DCR C

JNZ Loop

HLT

4-> Write a Program to Sort the array in ascending order/descending order.

Code:-

start: lxi H,1FFFH ; contains counter

mvi d,00H

mov c, m

dcr c

inx h

loop: mov a,m

inx h

cmp m ; compare with last element

jz next ; if equal no need to swap

jc next ; if less then no need to swap

;jnc next <===== comment above and uncomment for descending order

; greater then need to swap

mov b,m

mov m,a

dcx h

mov m,b

inx h

mvi d,01H ; set check swap happen

next: dcr c

jnz loop

mov a,d

cpi 01H ; if swap happen repeat the process

jz start

hlt

Before Sort

After Sort

5-> Write a Program to find Largest number in a given data array.

Code:-

LDA 0000H

MOV C, A

DCR C

LXI H, 0001H

MOV B, M

Loop: INX H

MOV A, M

CMP B

JC Here

MOV B, A

Here: DCR C

JNZ Loop

INX H

MOV M, B;

HLT

6-> Write a Program to move a block starting at location 2000H To location 3000H with overlap/without overlap.

Code:-

lxi h,1FFFH

mov c,m ;store block size

inx h ; first data byte of the source

lxi d,3000H ; destination address

mvi b,00H

dad b ; HL <- HL + BC(counter)

dcx h ; Point to the last of source

xchg

dad b

dcx h ; point to the last of destination

xchg

loop: mov a,m

stax d

dcx h

dcx d

dcr c

jnz loop

hlt