U18CO018

Shubham Shekhaliya

MIT

Assignment-8

Programming in 8086

1-> Write 8086 ALP for addition of two 8-bit numbers.

Code:-

model small

.data

ca db ?

p1 db ?

p2 db 80H

p3 db 44H

.code

mov ax, @data

mov ds, ax

mov cl, 00H

mov al, p2

add al, p3

jnc skip

inc cl

skip: mov p1, al

mov ca, cl

mov ax, 4C00H

int 21h

end

Output:-

2-> Write 8086 ALP for subtraction of two 8-bit numbers.

Code:-

model small

.data

ca db ?

p1 db ?

p2 db 42H

p3 db 44H

.code

mov ax, @data

mov ds, ax

mov cl, 00H

mov al, p2

sub al, p3

jnc skip

inc cl

skip: mov p1,al

mov ca, cl

mov ax, 4C00H

int 21h

end

Output:-

3-> Write 8086 ALP for addition of two 16-bit numbers

Code:-

model small

.data

ca db ?

p1 db ?

p2 db ?

p3 db 41H

p4 db 42H

p5 db 61H

p6 db 62H

.code

mov ax, @data

mov ds, ax

mov cx, 0000H

mov ah, p3

mov al, p4

mov bh, p5

mov bl, p6

add ax, bx

jnc skip

inc cx

skip: mov p1, ah

mov p2, al

mov ca, cl

mov ax, 4C00H

int 21H

end

Output:-

4-> Write 8086 ALP for subtraction of two 16-bit numbers

Code:-

model small

.data

ca db ?

p1 db ?

p2 db ?

p3 db 65H

p4 db 67H

p5 db 48H

p6 db 32H

.code

mov ax, @data

mov ds, ax

mov cx, 0000H

mov ah, p3

mov al, p4

mov bh, p5

mov bl, p6

sub ax, bx

jnc skip

inc cx

skip: mov p1, ah

mov p2, al

mov ca, cl

mov ax, 4c00H

int 21h

end

Output:-