U18CO018

Shubham Shekhaliya

MIT

Assignment-9

1-> Program to multiply signed 16-bit numbers.

Code:-

model small

.8086

.data

a dw 0004H

b dw -0002H

c dw 0

d dw 0

.code

mov ax, @data

mov ds, ax

mov ax, a

mov bx, b

imul bx

mov c, ax

mov d, dx

mov ax, 4C00H

int 21h

end

Output:-

2-> Program to multiply unsigned 16-bit numbers.

Code:-

model small

.8086

.data

a dw 0004H

b dw 0FFFEH

c dw 0

d dw 0

.code

mov ax, @data

mov ds, ax

mov ax, a

mov bx, b

mul bx

mov c, ax

mov d, dx

mov ax, 4C00H

int 21h

end

Output:-

3-> Program for division of unsigned 8-bit numbers.

Code:-

model small

.8086

.data

a db 28H

b db 03H

c dw ?

.code

mov ax, @data

mov ds, ax

mov ax,0000H

mov bx,0000H

mov al,a

mov bl,b

div bl

mov c,ax

mov ax, 4C00H

int 21h

end

Output:-

4-> Program for division of unsigned 16-bit numbers.

Code:-

model small

.8086

.data

a dw 0188H

b dw 0012H

c dw ?

d dw ?

.code

mov ax, @data

mov ds, ax

mov ax,0000H

mov bx,0000H

mov dx,0000H

mov ax,a

mov bx,b

div bx

mov c,ax

mov d,dx

mov ax, 4C00H

int 21h

end

Output:-

5-> Program for division of signed 8-bit numbers.

Code:-

model small

.8086

.data

a db 28H

b db -03H

c dw ?

.code

mov ax, @data

mov ds, ax

mov ax,0000H

mov bx,0000H

mov al,a

mov bl,b

idiv bl

mov c,ax

mov ax, 4C00H

int 21h

end

Output:-

6-> Program for division of signed 16-bit numbers.

Code:-

model small

.8086

.data

a dw 0188H

b dw -0012H

c dw ?

d dw ?

.code

mov ax, @data

mov ds, ax

mov ax,0000H

mov bx,0000H

mov dx,0000H

mov ax,a

mov bx,b

idiv bx

mov c,ax

mov d,dx

mov ax, 4C00H

int 21h

end

Output:-

7-> Program for data transfer using different addressing modes.

Code:-

model small

.8086

.data

s1 db "UVWXYZ$"

s2 db "ABCDEF$"

.code

mov ax, @data

mov ds, ax

mov ah,15h

mov bh,al

mov di,offset s1

mov bl,[di]

mov ax,[0012h]

mov si, offset s2

mov dl,[si+2]

in ax , 50H

mov ax, 4C00H

int 21h

end

Output :-

8-> Program to move data from source to destination using indirect addressing mode (Block Move without overlap).

Code:-

model small

.8086

.data

s1 db "UVWXYZ$"

s2 db 6 dup(0)

.code

mov ax, @data

mov ds, ax

mov si,offset s1

mov di,offset s2

mov cx,0006h

up: mov al,[si]

mov [di],al

inc si

inc di

dec cx

jnz up

mov ax, 4C00H

int 21h

end

Output :-

9-> Program to move a block of data from source to destination (With overlap in either direction.

Code:-

model small

.8086

.data

s1 db "UVWXYZ$"

.code

mov ax, @data

mov ds, ax

mov si,offset s1

mov cx,0006h

mov bl,05h

mov di,offset [s1+3]

up: inc si

inc di

dec bl

jnz up

go: mov al,[si]

mov [di],al

dec si

dec di

dec cx

jnz go

mov ax, 4C00H

int 21h

end

Before :-

After :-

10-> Program to interchange two blocks of data.

Code:-

model small

.8086

.data

s1 db "UVWXYZ$"

s2 db "123456$"

.code

mov ax, @data

mov ds, ax

mov si,offset s1

mov di,offset s2

mov cx,0006h

up: mov al,[si]

mov bl,[di]

mov [si],bl

mov [di],al

inc si

inc di

dec cx

jnz up

mov ax, 4C00H

int 21h

end

Before:-

After:-